Introduction to Communication Signal

- Signal and system:

Signals are time-varying quantities such as voltages or current.

A system is a combination of devices and networks made to perform a desired function.

- Classification of signals and system:

  - Continuous – time and discrete – time signals:

A signal is said to be continuous-time signals(t), when it is a real or complex function of time, where the independent variable t is continuous.

A signal is said to be discrete-time signals(t), when t is a discrete variable. A discrete-time signal is often identified as a sequence of numbers, denoted by sn, where n is an integer.

  - Analogue and digital signals:

A signal is said to be an analogue signal, when it can vary in a continuous time interval, then signalst, is called an analogue signal.

A signal is said to be a discrete-time signal, when it can take only a finite number of distinct values, sn, then the signal is called a digital signal.

  - Deterministic and random signals:

A signal is said to be a deterministic if it’s a completely specified function to time. A good example of a deterministic signal is a signal composed of a signal sinusoid, such as xt=Acos(2πf0t+ϕ).

With the signal parameters being:

A is the amplitude, f0is the frequency oscillation ratein cycles per second or hertz, and ϕ

is the phase in radians. depending on your background, you may be more familiar

with radian frequncy.

A signal said to be a random signal when their occurrence is random in nature. These signals cannot be specified in time. These signals are irregular in nature, that is, they do not repeat with time. Thermal noise is a type of random signal.

  - Periodic and non-periodic signal:

A signal st, is a periodic signal if st=st+nT0, where T0is called the period and the integer n>0.

A signal st, is a non-periodic signal if

st≠st+T0∀ t and any T0, then st

is a non-periodic or a periodic signal.

  - Power and energy signals:

Power signal: signals whose total power is finite and non-zero. The energy of the power signal will be infinite. Example: periodic sequences like sinusoid. A sinusoidal signal has finite, non-zero power but infinite energy. A signal cannot be both an energy signal and a power signal.

0<P=limT→∞-T2T/2sts*tdt<∞

A complex signal st, is an energy signal if the normalized energy E is finite, where

0<E=-∞∞sts*tdt=-∞∞st2dt<∞

Note: in communication systems, the received waveform is usually categorized into the desired part, containing the information signal, and the undesired part, called noise.

-------------------------------------------------------------------------------------------------------------

- Some useful functions:

  - Unit impulse function:

  - Unit step function:

  - Sampling function:

  - Sinc function:

  - Rectangular function:

  - Triangular function:

-------------------------------------------------------------------------------------------------------------

- Other useful operations:

  - Cross-correlation:

  - Auto-correlation:

  - Convolution: